RSV1000 / SL1000 ECU Fault Codes

To display fault codes:

1)  Remove right side panel (the piece that covers the rear subframe and lower gas tank) by removing the two 4mm button-head Allen bolts. Be careful of the plastic tabs on the backside of the panels.

2) You will find two unconnected wires near a big harness clip near the side of the battery. You cannot miss these wires. They are the only 2 unconnected wires in there.

3) Connect the two wires together.

4) Turn key  switch to "ON/RUN".

5) DO NOT start or attempt to start engine.

6) The message "DIAG" should appear on left LCD screen along with a number indicating the Fault Code.

7) If your bike is still under warranty, let your Aprilia dealer do all of this.

| Component | Fault Code | Possible Cause |
| --- | --- | --- |
| Camshaft position sensor | 11 | Sensor not connected. Sensor broken. Wiring faulty. |
| Crankshaft position sensor | 12 |  |
| Intake pressure sensor | 13 |  |
| Intake pressure sensor | 14 | Excessive pressure difference between two pressure sensors |
| Throttle position sensor | 15 | Sensor not connected. Sensor broken. Wiring faulty. |
| Coolant temperature sensor | 21 |  |
| Air temperature sensor | 22 |  |
| Atmospheric pressure sensor (inside ECU) | 23 | Sensor faulty |
| Ignition coil 1, Front cylinder | 33 | Ignition coil faulty or wiring not connected |
| Ignition coil 2, Front cylinder | 34 |  |
| Ignition coils 1 and 2, Front cylinder | 33,34 |  |
| Ignition coil 1, Rear cylinder | 35 |  |
| Ignition coil 2, Rear cylinder | 36 |  |
| Ignition coils 1 and 2, Rear cylinder | 35,36 |  |
| Fall Sensor (tip-over sensor) | 41 | Sensor not connected. Sensor broken. Wiring faulty. |

Aprilia and logo are registered trademarks of Aprilia, S.p.A., Italy. See Legal disclaimer.
apriliaforum.com assumes no liability for the use of these tips.  Consult your owners manual, the service manual, and your Aprilia dealer for more information. Use these tech tips at your own risk!